ARIA

AI Robot Intelligence Assistant

Complete Build & Setup Documentation

Balakrishna Reddy Channareddy  |  MS Business Analytics  |  University of the Pacific

# 1. Project Overview

ARIA (AI Robot Intelligence Assistant) is a fully autonomous AI robot built on Raspberry Pi Zero 2W and Arduino Uno. It combines computer vision, voice recognition, AI responses, and autonomous navigation.

## Key Features

- Real-time face detection and recognition

- Voice commands via Google Speech Recognition

- AI-powered responses via Claude API

- Pan-Tilt servo camera tracking

- Bluetooth speaker audio output (gTTS)

- Obstacle avoidance with HC-SR04 ultrasonic sensor

- iPhone following via HM-10 BLE module

- Autonomous 4WD navigation

# 2. Hardware Components

## Parts List

| Component | Model | Purpose |
| --- | --- | --- |
| Raspberry Pi Zero 2W | Pi Zero 2W (512MB) | Main AI brain |
| Arduino Uno | LAFVIN Arduino Uno R3 | Motor & sensor control |
| Camera | OV5647 5MP Night Vision | Face detection & tracking |
| Microphone | SuziePi USB Mini Mic | Voice input |
| Speaker | BTS001 Bluetooth Speaker | Audio output |
| Pan Servo | SG90 Servo Motor | Left/right camera tracking |
| Tilt Servo | SG90 Servo Motor | Up/down camera tracking |
| Motor Driver | L298N H-Bridge | Motor control |
| Ultrasonic | HC-SR04 | Obstacle detection |
| BLE Module | HM-10 Bluetooth 4.0 | iPhone following |
| Chassis | 4WD Robot Car Kit | Mobility platform |
| Power | Power Bank + USB Wall Charger | Pi & Arduino power |

# 3. Wiring Connections

## 3.1 Raspberry Pi GPIO Connections

| Component | Pi Pin | GPIO | Wire Color |
| --- | --- | --- | --- |
| Pan Servo Signal | Pin 12 | GPIO 18 | Orange |
| Tilt Servo Signal | Pin 16 | GPIO 23 | Orange |
| Both Servos VCC | Pin 2 | 5V | Red |
| Both Servos GND | Pin 6 | GND | Brown/Black |

## 3.2 Arduino Uno Pin Connections

| Component | Arduino Pin | Notes |
| --- | --- | --- |
| L298N ENA | Pin 5 (PWM) | Left motor speed |
| L298N ENB | Pin 9 (PWM) | Right motor speed |
| L298N IN1 | Pin 3 | Left motor direction A |
| L298N IN2 | Pin 4 | Left motor direction B |
| L298N IN3 | Pin 7 | Right motor direction A |
| L298N IN4 | Pin 8 | Right motor direction B |
| HC-SR04 TRIG | Pin 10 | Ultrasonic trigger |
| HC-SR04 ECHO | Pin 11 | Ultrasonic echo |
| HM-10 TX | Pin 2 | BLE receive |
| HM-10 RX | Pin 12 | BLE transmit |
| HM-10 VCC | 3.3V | MUST be 3.3V not 5V! |
| HM-10 GND | GND | Ground |

## 3.3 L298N Motor Driver Connections

| L298N Terminal | Connects To |
| --- | --- |
| OUT1, OUT2 | Left side motors (both) |
| OUT3, OUT4 | Right side motors (both) |
| 12V (VMS) | Battery pack positive |
| GND | Battery pack negative + Arduino GND |
| 5V output | NOT used (Arduino powered separately) |

IMPORTANT: Power Arduino via USB wall charger separately. Pi powered by power bank.

# 4. Project File Structure

/home/pi/robot/

├── main.py                    # Main ARIA brain

├── .env                       # API keys

├── requirements.txt           # Python packages

├── models/

│   ├── deploy.prototxt        # DNN face detector

│   └── res10_300x300_ssd.caffemodel

├── voice/

│   ├── listener.py            # Mic input

│   └── speaker.py             # Audio output

├── ai/

│   ├── assistant.py           # Claude API

│   └── weather.py             # Weather API

├── face_recognition/

│   ├── recognize.py           # Face detection & ID

│   ├── train.py               # Face training

│   └── known_faces/           # Saved face data (.pkl)

└── motor_control/

└── servo.py               # Pan-Tilt control

# 5. Raspberry Pi Setup

## 5.1 Flash OS & Configure WiFi

Step 1: Flash Raspberry Pi OS Legacy (32-bit) to SD card using Raspberry Pi Imager.

Step 2: Insert SD card in Mac and edit WiFi config:

# On Mac terminal:

cat > /Volumes/bootfs/network-config << 'EOF'

network:

version: 2

wifis:

wlan0:

dhcp4: true

optional: true

access-points:

"YourHotspotName":

password: "YourPassword"

EOF

## 5.2 SSH Into Pi

# Turn on iPhone hotspot first, then boot Pi

ssh pi@aria.local

# Password: raspberry

# If SSH key error:

ssh-keygen -R aria.local

ssh pi@aria.local

## 5.3 Install System Packages

sudo apt-get update

sudo apt-get install -y espeak flac mpg123 python3-pyaudio \

python3-opencv libopencv-dev

## 5.4 Install Python Packages

pip3 install anthropic speechrecognition gtts \

python-dotenv requests RPi.GPIO --break-system-packages

## 5.5 Clone Repository & Configure

git clone https://github.com/bala2207022/ARIA_robot ~/robot

cd ~/robot

# Create .env file with API keys

cat > .env << 'EOF'

CLAUDE_API_KEY=your_claude_api_key_here

WEATHER_API_KEY=your_openweathermap_key

NEWS_API_KEY=your_newsapi_key

EOF

## 5.6 Connect Bluetooth Speaker

sudo rfkill unblock bluetooth

sudo hciconfig hci0 up

bluetoothctl

# Inside bluetoothctl:

power on

agent on

scan on

# Wait for BTS0011 to appear, then:

pair 41:42:78:94:D0:DD

trust 41:42:78:94:D0:DD

connect 41:42:78:94:D0:DD

exit

## 5.7 Download Face Detection Model

mkdir -p ~/robot/models

wget -O ~/robot/models/deploy.prototxt \

"https://raw.githubusercontent.com/opencv/opencv/master/samples/dnn/face_detector/deploy.prototxt"

wget -O ~/robot/models/res10_300x300_ssd_iter_140000.caffemodel \

"https://raw.githubusercontent.com/opencv/opencv_3rdparty/dnn_samples_face_detector_20170830/res10_300x300_ssd_iter_140000.caffemodel"

## 5.8 Train Face Recognition

python3 ~/robot/face_recognition/train.py

# Enter your name when prompted

# Stand 3-4 feet from camera

# Move head slowly in different directions

# 6. Python Code Files

## 6.1 main.py

import time, os, threading, base64

import RPi.GPIO as GPIO

from voice.listener import listen

from voice.speaker import speak, is_speaking

from ai.assistant import ask_aria

from face_recognition.recognize import load_known_faces, capture_frame, recognize_face

from face_recognition.train import train_face

PAN_PIN = 18   # GPIO 18 - Left/Right

TILT_PIN = 23  # GPIO 23 - Up/Down

pan_angle = 90.0

tilt_angle = 60.0  # Start tilted up (robot on floor)

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

GPIO.setup(PAN_PIN, GPIO.OUT)

GPIO.setup(TILT_PIN, GPIO.OUT)

pan_pwm = GPIO.PWM(PAN_PIN, 50)

tilt_pwm = GPIO.PWM(TILT_PIN, 50)

pan_pwm.start(0)

tilt_pwm.start(0)

def move_pan(angle):

global pan_angle

angle = max(0, min(180, float(angle)))

if abs(angle - pan_angle) < 2: return

pan_pwm.ChangeDutyCycle(2.5 + (angle/180.0)*10)

time.sleep(0.15)

pan_pwm.ChangeDutyCycle(0)

pan_angle = angle

def move_tilt(angle):

global tilt_angle

angle = max(30, min(90, float(angle)))

if abs(angle - tilt_angle) < 2: return

tilt_pwm.ChangeDutyCycle(2.5 + (angle/180.0)*10)

time.sleep(0.15)

tilt_pwm.ChangeDutyCycle(0)

tilt_angle = angle

def track_face_servo(face_x, face_y):

global pan_angle, tilt_angle

x_off = face_x - 160

y_off = face_y - 120

if abs(x_off) > 25: move_pan(pan_angle - x_off * 0.04)

if abs(y_off) > 20: move_tilt(tilt_angle + y_off * 0.04)

# ... (see full code in repository)

See full code at: github.com/bala2207022/ARIA_robot

## 6.2 voice/listener.py

import speech_recognition as sr

import time

STOP_WORDS = ['stop', 'exit', 'quit', 'bye', 'goodbye']

def listen(wait_after_speak=1.5):

recognizer = sr.Recognizer()

recognizer.energy_threshold = 300

recognizer.dynamic_energy_threshold = True

recognizer.pause_threshold = 0.8

mic = None

for i in range(5):

try:

test = sr.Microphone(device_index=i)

with test as s:

recognizer.adjust_for_ambient_noise(s, duration=0.3)

mic = test

break

except: continue

if mic is None: return None

time.sleep(wait_after_speak)

with mic as source:

print('Listening...')

try:

audio = recognizer.listen(source, timeout=7, phrase_time_limit=8)

except sr.WaitTimeoutError: return None

try:

text = recognizer.recognize_google(audio)

print(f'You said: {text}')

if any(w in text.lower() for w in STOP_WORDS): return 'stop'

return text

except: return None

## 6.3 voice/speaker.py

import os, tempfile

is_speaking = False

def speak(text):

global is_speaking

is_speaking = True

print(f'ARIA: {text}')

try:

from gtts import gTTS

with tempfile.NamedTemporaryFile(suffix='.mp3', delete=False) as f:

tmp = f.name

tts = gTTS(text=text, lang='en', slow=False)

tts.save(tmp)

os.system(f'mpg123 -q {tmp} 2>/dev/null')

os.unlink(tmp)

except Exception as e:

safe = text.replace('"', "'")

os.system(f'espeak -s 145 "{safe}" 2>/dev/null')

is_speaking = False

## 6.4 ai/assistant.py

from datetime import datetime

import anthropic, os

from dotenv import load_dotenv

load_dotenv()

client = anthropic.Anthropic(api_key=os.getenv('CLAUDE_API_KEY'))

def ask_aria(question, recognized_name=None):

today = datetime.now().strftime('%A, %B %d, %Y')

current_time = datetime.now().strftime('%I:%M %p')

if recognized_name:

user_info = f'Person talking is {recognized_name}. Use their name naturally.'

else:

user_info = 'Do not know who is talking. Do not use any name.'

message = client.messages.create(

model='claude-sonnet-4-20250514',

max_tokens=150,

system=f'''You are ARIA, an AI Robot Intelligence Assistant.

Keep responses very short, max 2 sentences, spoken out loud.

Today is {today}. Time is {current_time}. Location: Stockton, CA.

{user_info}''',

messages=[{'role': 'user', 'content': question}]

)

return message.content[0].text

## 6.5 face_recognition/recognize.py

import cv2, pickle, os, numpy as np, subprocess, tempfile

HAARCASCADE = '/usr/share/opencv4/haarcascades/haarcascade_frontalface_default.xml'

FACES_DIR = os.path.join(os.path.dirname(os.path.abspath(__file__)), 'known_faces')

def capture_frame():

with tempfile.NamedTemporaryFile(suffix='.jpg', delete=False) as f:

tmp = f.name

subprocess.run(['rpicam-jpeg', '-o', tmp, '--timeout', '500',

'-n', '--width', '320', '--height', '240'], capture_output=True)

frame = cv2.imread(tmp)

try: os.unlink(tmp)

except: pass

return frame

def load_known_faces():

known_faces, known_names = [], []

os.makedirs(FACES_DIR, exist_ok=True)

for file in os.listdir(FACES_DIR):

if file.endswith('.pkl'):

with open(os.path.join(FACES_DIR, file), 'rb') as f:

data = pickle.load(f)

known_names.append(data['name'])

known_faces.append(data['images'])

return known_faces, known_names

def recognize_face(frame, known_faces, known_names):

face_cascade = cv2.CascadeClassifier(HAARCASCADE)

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

gray = cv2.equalizeHist(gray)

faces = face_cascade.detectMultiScale(gray, 1.05, 3, minSize=(30,30))

if len(faces) == 0:

faces = face_cascade.detectMultiScale(gray, 1.01, 1, minSize=(20,20))

results, positions = [], []

for (x, y, w, h) in faces:

positions.append((x+w//2, y+h//2))

face_img = cv2.resize(cv2.equalizeHist(gray[y:y+h, x:x+w]), (150,150))

best_score, best_name = float('inf'), 'Unknown'

for i, person_images in enumerate(known_faces):

for ki in person_images:

diff = np.mean(np.abs(face_img.astype(int)-cv2.resize(ki,(150,150)).astype(int)))

if diff < best_score: best_score, best_name = diff, known_names[i]

results.append(best_name if best_score < 150 else 'Unknown')

return results if results else ['No face'], positions

## 6.6 face_recognition/train.py

import cv2, pickle, os, subprocess, tempfile, time

HAARCASCADE = '/usr/share/opencv4/haarcascades/haarcascade_frontalface_default.xml'

FACES_DIR = os.path.join(os.path.dirname(os.path.abspath(__file__)), 'known_faces')

def train_face(name, speak_fn=None):

os.makedirs(FACES_DIR, exist_ok=True)

if os.path.exists(os.path.join(FACES_DIR, f'{name}.pkl')):

if speak_fn: speak_fn(f'I already know you {name}!')

return True

face_cascade = cv2.CascadeClassifier(HAARCASCADE)

images, count, attempts = [], 0, 0

prompts = ['Look straight','Tilt left','Tilt right','Look up','Look down']

if speak_fn: speak_fn(f'Okay {name}! I will take 20 photos.')

while count < 20 and attempts < 100:

attempts += 1

if count % 4 == 0 and count//4 < len(prompts):

if speak_fn: speak_fn(prompts[count//4])

time.sleep(1)

# capture frame and detect face...

# (see full code in repository)

if count >= 15:

data = {'name': name, 'images': images}

with open(os.path.join(FACES_DIR, f'{name}.pkl'), 'wb') as f:

pickle.dump(data, f)

if speak_fn: speak_fn(f'Done! I will remember you {name}!')

return True

return False

# 7. Arduino Code

## 7.1 Upload Instructions

1. Connect Arduino to Mac via USB cable

2. Open Arduino IDE

3. Select Board: Arduino UNO

4. Select Port: /dev/cu.usbmodem... (whatever appears)

5. DISCONNECT HM-10 TX/RX wires before uploading

6. Paste code below and click Upload arrow

7. After upload, reconnect HM-10 wires

## 7.2 Complete Arduino Code

#include <SoftwareSerial.h>

#define ENA 5

#define ENB 9

#define IN1 3

#define IN2 4

#define IN3 7

#define IN4 8

#define TRIG 10

#define ECHO 11

SoftwareSerial BTSerial(2, 12); // HM-10: RX=pin2, TX=pin12

String input = "";

bool phoneNear = false;

String lastAction = "";

void stopMotors() {

if (lastAction == "STOP") return;

analogWrite(ENA, 0); analogWrite(ENB, 0);

digitalWrite(IN1, LOW); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW); digitalWrite(IN4, LOW);

lastAction = "STOP";

}

void forward() {

if (lastAction == "FORWARD") return;

analogWrite(ENA, 200); analogWrite(ENB, 200);

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

lastAction = "FORWARD";

}

void backward() {

analogWrite(ENA, 200); analogWrite(ENB, 200);

digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH);

digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH);

lastAction = "BACKWARD";

}

void turnRight() {

analogWrite(ENA, 180); analogWrite(ENB, 180);

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);  digitalWrite(IN4, HIGH);

lastAction = "RIGHT";

}

void turnLeft() {

analogWrite(ENA, 180); analogWrite(ENB, 180);

digitalWrite(IN1, LOW);  digitalWrite(IN2, HIGH);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

lastAction = "LEFT";

}

long getDistance() {

digitalWrite(TRIG, LOW); delayMicroseconds(2);

digitalWrite(TRIG, HIGH); delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long duration = pulseIn(ECHO, HIGH, 30000);

if (duration == 0) return 999;

return duration * 0.034 / 2;

}

void setup() {

Serial.begin(9600);

BTSerial.begin(9600);

pinMode(ENA, OUTPUT); pinMode(ENB, OUTPUT);

pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT);

pinMode(TRIG, OUTPUT); pinMode(ECHO, INPUT);

stopMotors();

Serial.println("ARIA Ready!");

}

void loop() {

while (BTSerial.available()) {

char c = BTSerial.read();

if (c == '\n' || c == '\r') continue;

input += c;

delay(10);

}

if (input.length() > 0) {

input.trim(); input.toLowerCase();

if (input == "near")     phoneNear = true;

else if (input == "far") phoneNear = false;

else if (input == "f")  { lastAction=""; forward(); }

else if (input == "b")  { lastAction=""; backward(); }

else if (input == "l")  { lastAction=""; turnLeft(); }

else if (input == "r")  { lastAction=""; turnRight(); }

else if (input == "s")  { lastAction=""; stopMotors(); }

input = "";

}

long distance = getDistance();

if (phoneNear) {

stopMotors();

} else {

if (distance < 20 && distance > 0) {

stopMotors(); delay(300);

backward(); delay(400);

stopMotors(); delay(200);

long dL, dR;

turnLeft(); delay(300); stopMotors();

dL = getDistance();

turnRight(); delay(600); stopMotors();

dR = getDistance();

if (dL > dR) { turnLeft(); delay(300); }

else { turnRight(); delay(300); }

stopMotors(); delay(200); lastAction = "";

} else { forward(); }

}

delay(100);

}

# 8. Voice Commands

| Say This | ARIA Does This |
| --- | --- |
| "Hello ARIA" | ARIA greets you |
| "Who are you" | ARIA introduces itself |
| "Who am I" | ARIA scans face and identifies you |
| "Remember me" / "Train me" | ARIA learns your face (20 photos) |
| "What do you see" | Camera analyzes scene via Claude Vision |
| "What is this" | ARIA describes what camera sees |
| "What's the weather" | Current weather for Stockton, CA |
| "What time is it" | Current time |
| "What's the date" | Current date |
| Any question | Claude AI answers intelligently |
| "Goodbye" / "Bye" | ARIA shuts down |

# 9. Running ARIA

## Step-by-Step Startup

1. Turn on iPhone hotspot (SSID: Bala, Password: 12345678)

2. Plug power bank into Pi PWR IN port

3. Wait 60 seconds for Pi to boot and connect

4. Power Arduino via USB wall charger

5. SSH into Pi:

ssh pi@aria.local  # password: raspberry

6. Connect Bluetooth speaker:

bluetoothctl

connect 41:42:78:94:D0:DD

exit

7. Run ARIA:

cd ~/robot && python3 main.py

# 10. Troubleshooting

| Problem | Solution |
| --- | --- |
| SSH not connecting | Turn hotspot off/on. Run: ssh-keygen -R aria.local |
| Bluetooth not connecting | Run: sudo rfkill unblock bluetooth && sudo hciconfig hci0 up |
| Speaker no sound | Reconnect: bluetoothctl → connect 41:42:78:94:D0:DD |
| Camera not detecting face | Stand 3-4 feet away, good lighting, face camera directly |
| Servo not moving | Check GPIO 18/23 wiring. Run servo test script |
| Mic not responding | Speak loudly and clearly, mic device_index=0 |
| AI not responding | Check .env has correct CLAUDE_API_KEY |
| Motors not moving | Check L298N wiring and battery connection |
| Arduino upload fails | Disconnect HM-10 TX/RX wires before uploading |
| cv2.data error | Use path: /usr/share/opencv4/haarcascades/ |

# 11. Future Improvements

- HC-05/HM-10 automatic RSSI-based phone following

- Wide-angle camera lens for better field of view

- Offline voice recognition using Vosk

- GPS-based outdoor navigation

- Mobile app for remote ARIA control

- Multi-person face recognition

- Emotion detection from facial expressions

- Spotify music control integration

- Home automation integration

ARIA Project  |  Balakrishna Reddy Channareddy  |  github.com/bala2207022/ARIA_robot